- Four Forces Acting on an Airplane

- Principles of Flight  - Principle of Differential Pressure

- Daniel Bernoulli – Swiss Mathematician 1730

Typical Airfoil

Relative Wind

Angle of attach (AOA)

Higher pressure below, lower pressure above

All lift is said to act at the Center of Pressure (CP)

Higher AOA creates larger differential pressure – more lift

Higher speed creates larger diff pressure – more lift

Penalty of creating lift is drag – Induced Drag

Penalty of moving wing and airplane thru the air is drag – Parsite Drag

Product of creating lift is drag – Creation of Wing Tip Vortices around wing tip

Wake Turbulence Generation

Drag	PHOAK 5-6

  - Parasite Drag		PHOAK 5-6

    - Form		PHOAK 5-6

    - Interference	PHOAK 5-6

    - Skin Friction	PHOAK 5-6

  - Induced Drag	PHOAK 5-7

- Axis of an Airplane

- Flight Controls

Primary:	Ailerons, Elevators, Rudder

Secondary:	Flaps, Trim

- Back to 4 Forces

Four forces are equal in unaccelerated flight

Level, climbing, descending

Pilot controls amount of lift in two ways.

AOA		lift is proportional to AOA

Airspeed	lift varies with square of the airspeed

Only so much AOA – critical angle of attack – STALL

- What makes an airplane turn

- Horizontal component of lift

- Adverse Yaw

- Load Factor

@45o	 LF=1.4

20% increase in stall speed

48 kts > 57 kts

42 kts > 50 kts

@50o	 LF=1.5

27% increase in stall speed

48 kts > 61 kts

42 kts > 54 kts

@60o	 LF=2.0

42% increase in stall speed

48 kts > 68 kts

42 kts > 60 kts

- Back to AOA

Higher AOA = higher diff press,  therefore higher lift

Critical angle of attack (AOA)  - WING STALLS

Two ways to create more lift:

Increase AOA

Increase Airspeed

AOA doubles 		- lift doubles

Airspeed doubles	- lift quaduples

Penalty of lift is induced drag

Parasite drag vs Induced Drag

- Wingtip Vortices

- Ground Effect

- Left Turning Tendency

Caused by:	1)  Torque reaction from engine and propeller

2)  Corkscrewing effect of the slipstream

3)  Gyroscopic action of the propeller

4)  Asymmetric loading of the propeller (P Factor)

Left Turning Tendency  (con’t)

- Stability

Static  - positive, neutral, negative

Dynamic – positive, neutral, negative

- Lateral & Longitudinal Stability – W & B

- Weight & Balance – Longitudinal Stability

Maneuvering Speed (Va)

Maneuvering Speed (Va)

Maneuvering Speed (Va)

Airplane flying at 110 kts, gross weight = 2,500 #’s,  AOA =  4.5o

If critical AOA is 18o, at 110 kts airplane will only experience 4 G’s before reaching critical AOA

Maneuvering Speed (Va)

Same airplane flying at 110 kts, gross weight = 1,666 #’s,  AOA = 3.0o

Critical AOA is still 18o, at 110 kts the airplane can now experience 6 G’s

before reaching critical AOA, which would exceed the max design load.

Maneuvering Speed (Va)

Same airplane flying at 95 kts, gross weight = 1,666 #’s,  AOA is now = 4.5o

At this lighter gross weight, if critical AOA is 18o, at 95 kts airplane will only

experience 4 G’s before reaching critical AOA.